# Installation Notes

CLC Firmware 0.1.27

Prior to installing, it is recommended to review the CLC Firmware 0.1.27 Release Notes. The following are the instruction to install the firmware update.

# Accessing CLC Firmware

The website is the primary means for delivering firmware.  If a CD is desired, one can be requested by contacting DSI Customer Service (CService@datasci.com).

- Navigate to www.datasci.com

- Browse to Resources | DSI Gateway Portal.

- Login using your User Name and Password. If you have forgotten or do not have a User Name and Password, the ability to request a new password or register at the site is available.

- Select Software Updates.

- Click the CLC 0.1.27.zip to download the contents to a desired location.

- Extract the files by right-clicking the .zip file and choosing Extract.

# Installing CLC Firmware

- With the CLC connected to the system, launch Ponemah.

- Select Hardware | PhysioTel Digital Diagnostics…
Note: If using Ponemah v5.20, select Hardware | Diagnostics…

- Choose the CLC from the Name – Model Name (Serial Number) dropdown.
Note: Select the Refresh List button if the CLC does not appear in the dropdown list.

- Select the Upload CLC Firmware link from the list view on the left side of diagnostics page.

- Please complete steps a. – d. below twice.

  - Select the Browse… button.

  - Select the CLC 0.1.27 file obtained during the .zip extract step above. Once uploaded, the following message will appear: Validated. Upgrade will be applied during reboot.

  - Select the reboot link within the message to navigate to the Reboot diagnostics page.

  - Select the Reboot button.